University of Mumbai

Examination 2020- Inter Cluster

Program: BE Instrumentation Engineering

Curriculum Scheme: Revised 2016

Examination:  Final Year Semester VIII

Course Code and Course Name: ISDLO8044 Power Plant Instrumentation

Time: 1hour                                                                                Max. Marks: 50

=====================================================================

Name of the Convener: Pramod B. Gawande

College Name: RGIT

Mobile Number: 9930007066

Email Id: pramod.gawande2810@gmail.com

======================================================================

Note to the students:- All Questions are compulsory and carry equal marks .

| Q1. | Coal is carried from storage to the boiler by means of? |
| --- | --- |
| Option A: | Trolleys |
| Option B: | V – belts |
| Option C: | Buckets |
| Option D: | Manually |
| Q2. | In case of non-condensing turbines the back pressure is |
| Option A: | Atmospheric |
| Option B: | below atmospheric pressure |
| Option C: | above atmospheric pressure |
| Option D: | Depends on the size of turbine. |
| Q3. | In a throttling process |
| Option A: | Steam temperature remains constant |
| Option B: | Steam pressure remains constant |
| Option C: | Steam enthalpy remains constant |
| Option D: | Steam entropy remains constant |
| Q4. | The flow through a nozzle is regarded as |
| Option A: | Constant volume flow |
| Option B: | Constant pressure flow |
| Option C: | Isothermal flow |
| Option D: | Isentropic flow |
| Q5. | The steam enters the nozzle at a |
| Option A: | High pressure and a low velocity |
| Option B: | High pressure and a high velocity |
| Option C: | Low pressure and a low velocity |
| Option D: | Low pressure and a high velocity |
| Q6. | Sulphur content of fuels is very important to the plant operators because it |
| Option A: | Has high heating value |
| Option B: | Retards electric precipitation |
| Option C: | Promotes complete combustion |
| Option D: | Has highly corrosive effect |
| Q7. | Which is not correct statement about the function of furnace? |
| Option A: | To provide proper conditions for continuous complete combustion |
| Option B: | Mix fuel with air and ignite |
| Option C: | Separate ash from coal |
| Option D: | Maintain heat supply to prepare and ignite the incoming fuel |
| Q8. | In a glass tube type water indicator for a boiler, one end of the tube is connected to water space and the other end is connected to |
| Option A: | Water space also |
| Option B: | Chimney |
| Option C: | Steam space |
| Option D: | Superheater |
| Q9. | In a velocity compounded impulse turbine, when steam flows through the second row of moving blades, |
| Option A: | Velocity increases |
| Option B: | Velocity decreases |
| Option C: | Velocity remains constant |
| Option D: | Pressure remains constant |
| Q10. | Efficiency of a thermal cycle increases by |
| Option A: | Regeneration |
| Option B: | Reheating of steam |
| Option C: | Both (A) and (B) |
| Option D: | Cooling of steam |
| Q11. | The air pressure at the fuel bed is reduced below that of atmosphere by means of a fan placed at or near the bottom of the chimney to produce a draught. Such a draught is called |
| Option A: | Natural draught |
| Option B: | Induced draught |
| Option C: | Forced draught |
| Option D: | Balanced draught |
| Q12. | The increase in pressure |
| Option A: | Lowers the boiling point of a liquid |
| Option B: | Raises the boiling point of a liquid |
| Option C: | Does not affects the boiling point of a liquid |
| Option D: | Reduces its volume |
| Q13. | A stage, in reaction turbine, is represented by |
| Option A: | Number of casing |
| Option B: | Number of entries of steam |
| Option C: | Number of exits of steam |
| Option D: | Each row of blades |
| Q14. | The diameter of tubes for natural circulation boiler as compared to controlled circulation boilers is |
| Option A: | More |
| Option B: | Less |
| Option C: | Same |
| Option D: | Could be more or less depending on other factors |
| Q15. | An economizer in a boiler |
| Option A: | Increases steam pressure |
| Option B: | Increases steam flow |
| Option C: | Decreases fuel consumption |
| Option D: | Decreases steam pressure |
| Q16. | Which is not correct statement about pulverized fuel firing? |
| Option A: | High burning rate is possible |
| Option B: | Heat release can be easily controlled |
| Option C: | Fuel burns economically |
| Option D: | It is the best technique for burning high ash content fuel having low fusion ash |
| Q17. | The process of maintaining the speed of the turbine constant for various load conditions, is known as |
| Option A: | Bleeding |
| Option B: | Reheating |
| Option C: | Governing |
| Option D: | Superheating |
| Q18. | The pressure of feed water has to be raised before its entry into the boiler. The pressure is raised by a device known as |
| Option A: | Feed pump |
| Option B: | Injector |
| Option C: | Feed check valve |
| Option D: | Pressure gauge |
| Q19. | Compounding of steam turbine is done for |
| Option A: | reducing the work done |
| Option B: | increasing the rotor speed |
| Option C: | reducing the rotor speed |
| Option D: | balancing the turbine |
| Q20. | The draught which a chimney produces is called |
| Option A: | Induced draught |
| Option B: | natural draught |
| Option C: | forced draught |
| Option D: | balanced draught |
| Q21. | For the induced draught the fan is located |
| Option A: | near bottom of chimney |
| Option B: | near bottom of furnace |
| Option C: | at the top of the chimney |
| Option D: | anywhere permissible |
| Q22. | In jet type condensers |
| Option A: | cooling water passes through tubes and steam surrounds them |
| Option B: | steam passes through tubes and cooling water surrounds them |
| Option C: | steam and cooling water mix |
| Option D: | steam and cooling water do not mix |
| Q23. | Pipes carrying steam are generally made up of |
| Option A: | Steel |
| Option B: | cast iron |
| Option C: | Copper |
| Option D: | Aluminium |
| Q24. | What is the pH value of water permissible for boiler? |
| Option A: | 0 |
| Option B: | 7 |
| Option C: | slightly less than 7 |
| Option D: | slightly more than 7 |
| Q25. | The working fluid in liquid phase of a steam power plant is: |
| Option A: | Water |
| Option B: | Graphite |
| Option C: | Steam |
| Option D: | Flue gases |
| Q26. | The following is the correct order of energy conversion in thermal power plants |
| Option A: | Chemical energy – Mechanical energy – Electrical energy |
| Option B: | Mechanical energy – Chemical energy – Electrical energy |
| Option C: | Wind energy – Mechanical energy – Electrical energy |
| Option D: | Heat energy – Electrical energy – Mechanical energy |
| Q27. | The path of flue gases in Thermal power plant is |
| Option A: | Boiler – Economizer – Superheater– Air preheater |
| Option B: | Boiler – Superheater – Air preheater – Economizer |
| Option C: | Boiler – Air preheater – Superheater – Economizer |
| Option D: | Boiler – Superheater – Economizer – Air preheater |
| Q28. | The following is not a component of Thermal power plant |
| Option A: | Condenser |
| Option B: | Cooling tower |
| Option C: | Turbine |
| Option D: | Fuel tank |
| Q29. | A ‘stroker’ is a power operated fuel ___ mechanism |
| Option A: | Burning |
| Option B: | Feeding |
| Option C: | Handling |
| Option D: | Storage |
| Q30. | The following is not a pulverized fuel burner. |
| Option A: | Tangential burner |
| Option B: | Turbulent burner |
| Option C: | Cyclone burner |
| Option D: | Radial burner |
| Q31. | The following is dry type dust collectors |
| Option A: | Spray type |
| Option B: | Packed type |
| Option C: | Impingement type |
| Option D: | cyclone separator |
| Q32. | Which fuel dominates the energy mix in Indian energy scenario? |
| Option A: | Oil |
| Option B: | Natural gas |
| Option C: | Coal |
| Option D: | nuclear |
| Q33. | The fourth largest producer of coal and lignite in the world is ______ |
| Option A: | USA |
| Option B: | Russia |
| Option C: | India |
| Option D: | china |
| Q34. | Indian per capita energy consumption is ____ of the world average. |
| Option A: | 4% |
| Option B: | 20% |
| Option C: | 1% |
| Option D: | 10% |
| Q35. | Energy consumption per unit of Gross Domestic Product is called as: |
| Option A: | Energy Ratio |
| Option B: | Energy intensity |
| Option C: | Per capita consumption |
| Option D: | None |
| Q36. | What is a load factor? |
| Option A: | The ratio of average to maximum demand. |
| Option B: | The ratio of maximum demand to average load. |
| Option C: | The product of maximum demand and average load. |
| Option D: | The ratio of average load to the plant capacity. |
| Q37. | Diversity factor is helpful in computing which of the following factors? |
| Option A: | Average load. |
| Option B: | Plant capacity |
| Option C: | Units generated (kWh) |
| Option D: | Peak demand. |
| Q38. | An industrial consumer has a load pattern of 2000 kW 0.8 lag for 12 hours and 1000 kW unity power factor for 12 hours. What is the load factor? |
| Option A: | 0.5 |
| Option B: | 0.55 |
| Option C: | 0.6 |
| Option D: | 0.75 |
| Q39. | What is the plant capacity factor? |
| Option A: | A ratio of kWh generated to the product of plant capacity and the number of hours for which the plant is in operation. |
| Option B: | The ratio of sum of individual maximum demands to the maximum demand on power stations. |
| Option C: | The ratio of actual energy produced to the maximum possible energy. |
| Option D: | The ratio of maximum demand on the power station to the connected load. |
| Q40. | Capacity factor will be very low when the power plant |
| Option A: | Is operated as base load plant. |
| Option B: | Is operated for supplying base load as well as the peak loads. |
| Option C: | Is operated in emergency only. |
| Option D: | Is under maintainance. |
| Q41. | When a pumped storage power plant is operated in conjunction with a steam power plant |
| Option A: | operating cost of steam plant becomes optimum |
| Option B: | Load factor of the steam plant is increased |
| Option C: | The chances of the tripping of the system decrease because of the use of two heterogeneous types of plants. |
| Option D: | Operation of the storage plant in the pumping mode during the low load period improves the steam plant stability. |
| Q42. | The Term Surge Tank is associated with |
| Option A: | Low Head Hydro |
| Option B: | Medium head hydro |
| Option C: | Thermal plant |
| Option D: | High head hydro |
| Q43. | With reference to hydropower station the graphical representation of the discharge as a function of time is known as: |
| Option A: | Monograph |
| Option B: | Load duration Curve |
| Option C: | Hectograph |
| Option D: | Hydro graphical |
| Q44. | Low head hydro plant is also known as ____________ |
| Option A: | Canal power plant |
| Option B: | Medium head hydro-plant |
| Option C: | Run-off river hydro-plant |
| Option D: | Base load hydro plant |
| Q45. | Which plants supply the peak load for the base power plants? |
| Option A: | Mini hydro plants |
| Option B: | Pump storage power plants |
| Option C: | Low head plants |
| Option D: | Run-off river power plants |
| Q46. | Which plants are used with steam and IC engines? |
| Option A: | Pumped storage plants |
| Option B: | Mini hydro plants |
| Option C: | Low head plants |
| Option D: | Run-off river power plants |
| Q47. | Which type of turbines does modern hydro power plant use? |
| Option A: | Kaplan turbine |
| Option B: | Francis turbine |
| Option C: | Pelton wheel |
| Option D: | Cross flow turbine |
| Q48. | Which type of hydro power plant can be with or without pondage? |
| Option A: | Pumped storage plants |
| Option B: | Mini hydro plants |
| Option C: | Low head plants |
| Option D: | Run-off river power plants |
| Q49. | The surge tank controls the water when the load on the turbine is ________ |
| Option A: | Equal |
| Option B: | Decreased |
| Option C: | Increased |
| Option D: | Not present |
| Q50. | Which type of valves is preferred for moderate heads? |
| Option A: | Butterfly valve |
| Option B: | Tube valve |
| Option C: | Needle vale |
| Option D: | Globe valve |
| Q51. | Which type of gate valves are used in high head installations? |
| Option A: | Needle valves |
| Option B: | Butterfly valves |
| Option C: | Globe valve |
| Option D: | Pinch valve |
| Q52. | Francis, kaplan and propeller turbines fall under the category of....... |
| Option A: | Impulse turbine |
| Option B: | Reaction turbine |
| Option C: | Impulse reaction combined |
| Option D: | Axial flow |
| Q53. | Where is the draft tube of a hydropower station that is an airtight pipe located. |
| Option A: | Near the surge tank. |
| Option B: | In between the penstock and the runner. |
| Option C: | In between the runner exhaust and the tailrace. |
| Option D: | At the beginning of penstock. |
| Q54. | In hydropower station what is an enlarged body of water just above the intake and used as a regulating reservoir called? |
| Option A: | pillways. |
| Option B: | Forebay |
| Option C: | Reservoir |
| Option D: | Penstock |
| Q55. | Boiling water reactor employs |
| Option A: | Boiler |
| Option B: | Direct cycle of coolant cycle |
| Option C: | Multi pass system |
| Option D: | Single Pass system |
| Q56. | Which of the following nuclear reactor does not need a heat exchanger for generation of steam? |
| Option A: | Gas cooled |
| Option B: | Liquid metal cooled |
| Option C: | Pressurised water |
| Option D: | Boiling water |
| Q57. | Half life of a radioactive isotope corresponds to the time required for half of the following to decay |
| Option A: | Electrons |
| Option B: | Protons |
| Option C: | Neutrons |
| Option D: | Atom |
| Q58. | Why has nuclear energy become an inevitable option for the development of the country? |
| Option A: | Because less pollution caused by nuclear plant |
| Option B: | High efficiency of nuclear energy |
| Option C: | Due to acute shortage of other sources of energy |
| Option D: | High cost of energy production of other sources |
| Q59. | How much amount of nuclear energy burnt is equivalent to the energy produced by 3000 tonnes of coal? |
| Option A: | 1kg |
| Option B: | 5kg |
| Option C: | 15kg |
| Option D: | 20kg |
| Q60. | What is the most attractive part of nuclear energy? |
| Option A: | Supports countries development |
| Option B: | Causes no pollution |
| Option C: | Has high efficiency of energy production |
| Option D: | Is available in abundance |
| Q61. | Nucleus consists of two sub-particles known as? |
| Option A: | Nucleotides |
| Option B: | Nucleons |
| Option C: | Neutrons |
| Option D: | Nucleosides |
| Q62. | The net neutrons produced per initial neutron accounting for all possible losses is called? |
| Option A: | Bombardment |
| Option B: | Half life |
| Option C: | Multiplication factors |
| Option D: | Covalent bond |
| Q63. | What is the time during which one half of a number of radioactive species decays or one half of their activity ceases? |
| Option A: | - Half life |
| Option B: | - Super critical state |
| Option C: | - Semi life |
| Option D: | - Critical life |
| Q64. | Electrons that orbit outermost shell of an atom are called? |
| Option A: | - Valence electrons |
| Option B: | - Electrons |
| Option C: | Electron coefficients |
| Option D: | - Neutrons |
| Q65. | A covalent bond is also called as ____________ |
| Option A: | Atomic bond |
| Option B: | Metal bond |
| Option C: | Molecular bond |
| Option D: | Metal bond |
| Q66. | PWR stands for ________ |
| Option A: | Pacific weather Report |
| Option B: | Partially weathered rock |
| Option C: | Pressurized water Reactor |
| Option D: | Packaging waste regulations |
| Q67. | What does the top of the pressurizer in pressurized water reactor consists of at primary system pressure? |
| Option A: | Steam |
| Option B: | Air |
| Option C: | Water |
| Option D: | Fluids |
| Q68. | LWR stands for _________ |
| Option A: | Lower water reactor |
| Option B: | Line water reactor |
| Option C: | Liquefied water reactor |
| Option D: | Light water reactor |
| Q69. | ### Which reactor consists of both fertile and fissile material? |
| Option A: | Fast breeder reactor |
| Option B: | Pressurize water reactor |
| Option C: | Boiling Water reactor |
| Option D: | Converter reactor |
| Q70. | The control rods in the control system of nuclear reactors are used to |
| Option A: | Absorb excess neutrons |
| Option B: | Control fuel consumption |
| Option C: | Control temperature |
| Option D: | Control pressure |
| Q71. | A moderator, in nuclear power plants, is a medium introduced into the fuel mass in order to |
| Option A: | Slow down the speed of fast moving neutrons |
| Option B: | Control the reaction |
| Option C: | Reduce the temperature |
| Option D: | Extracts heat from nuclear reaction |
| Q72. | In nuclear fission each neutron that causes fission releases |
| Option A: | No new neutron |
| Option B: | At least one new neutron |
| Option C: | One new neutron |
| Option D: | More than one new neutron |
| Q73. | A nuclear fission is initiated when the critical energy as compared to neutron binding energy of the atoms is |
| Option A: | Same |
| Option B: | More |
| Option C: | Less |
| Option D: | There is no such criterion |
| Q74. | Which is the more economical way of transporting coal? |
| Option A: | Sea or River ways |
| Option B: | Railways |
| Option C: | Road transporting |
| Option D: | By airlifting |
| Q75. | ________________Most efficient power plant |
| Option A: | Hydropower plant |
| Option B: | Nuclear power plant |
| Option C: | Thermal power plant |
| Option D: | Diesel power plant |
| Q76. | ______________Has highest initial cost |
| Option A: | Hydropower plant |
| Option B: | Nuclear power plant |
| Option C: | Thermal power plant |
| Option D: | Diesel power plant |
| Q77. | ____________Is most simple and clean |
| Option A: | Hydropower plant |
| Option B: | Nuclear power plant |
| Option C: | Thermal power plant |
| Option D: | Diesel power plant |
| Q78. | The steam power plant serves as a base plant for ________ |
| Option A: | Nuclear power plant |
| Option B: | Geothermal power plant |
| Option C: | Thermal power plant |
| Option D: | Diesel plant |
| Q79. | Which of the following least affect the cost of electricity produced in thermal power plant? |
| Option A: | Cost of fuel |
| Option B: | Cost of transportation |
| Option C: | The load factor |
| Option D: | Salaries of higher officials |
| Q80. | Which of the following power plant have longest physical life? |
| Option A: | Thermal power plant |
| Option B: | Nuclear power plant |
| Option C: | Hydroelectric power plant |
| Option D: | Diesel power plant |
| Q81. | Electron volt is the unit of |
| Option A: | Atomic power |
| Option B: | Energy |
| Option C: | Voltage |
| Option D: | Radio activity |
| Q82. | Which of the following power plants can be profitably employed for supplying base loads as well as peak loads? |
| Option A: | Thermal power plant |
| Option B: | Nuclear power plant |
| Option C: | Hydroelectric power plant |
| Option D: | Diesel power plant |
| Q83. | Which of following power plants normally operage at high speeds |
| Option A: | Hydro electric. |
| Option B: | Thermal. |
| Option C: | Diesel engine. |
| Option D: | Petrol engine. |
| Q84. | Where reactor operation is designed with fast neutrons such as in reactors using highly enriched fuel, the moderator used is |
| Option A: | Heavy water |
| Option B: | Graphite |
| Option C: | Carbon dioxide |
| Option D: | No moderator is needed |
| Q85. | What does WECS stands for? |
| Option A: | Wind energy conversion system |
| Option B: | Wind engine control system |
| Option C: | Wind energy combined system |
| Option D: | Wind engine comparison system |
| Q86. | In which of the following, does machine rotor drives through a step up gear box? |
| Option A: | Horizontal axis with two aerodynamic blades |
| Option B: | Horizontal axis propeller type wind mill |
| Option C: | Horizontal axis multi bladed type wind mill |
| Option D: | Sail type wind mill |
| Q87. | Which windmill blades are made by an array of wooden slats? |
| Option A: | Horizontal axis with two aerodynamic blades |
| Option B: | Horizontal axis propeller type wind mill |
| Option C: | Horizontal axis multi bladed type wind mill |
| Option D: | Horizontal axis wind mills Dutch type |
| Q88. | With upto how many propellers can windmills are built? |
| Option A: | 4 |
| Option B: | 2 |
| Option C: | 7 |
| Option D: | 6 |
| Q89. | Select the formula for total power Pt? |
| Option A: | Pt = 1/2gc ρAVi3 |
| Option B: | Pt = ρAVi3D3 |
| Option C: | Pt = 1/2gc Vi3D3 |
| Option D: | Pt = 2gcVi3 |
| Q90. | Which type of windmill blades are made out of sheet metal or aluminum? |
| Option A: | Horizontal axis with two aerodynamic blades |
| Option B: | Horizontal axis propeller type wind mill |
| Option C: | Horizontal axis multi bladed type wind mill |
| Option D: | Sail type wind mill |
| Q91. | A device which is used as device for showing direction wind as well used as a decorative purpose? |
| Option A: | Wind socks |
| Option B: | Weather vane |
| Option C: | Pin wheels |
| Option D: | Anemometers |
| Q92. | Turbines with how many propellers are used in order to avoid vibrations? |
| Option A: | 1 |
| Option B: | 2 |
| Option C: | 3 |
| Option D: | 4 |
| Q93. | Which type of windmill has better performance? |
| Option A: | Vertical type wind mills |
| Option B: | Darrieus type machines |
| Option C: | Magnus effect rotor |
| Option D: | Horizontal type windmills |
| Q94. | Which type of vertical wind machine consists of spinning cylinders? |
| Option A: | Cup anemometer |
| Option B: | Savonius rotor |
| Option C: | Darrieus type rotor |
| Option D: | Magnus effect rotor |
| Q95. | What is the total power produced if the turbine diameter is 90m |
| Option A: | 0.155KW |
| Option B: | 0.982 KW |
| Option C: | 1.452 KW |
| Option D: | 3.12 KW |
| Q96. | Global Cold wind generated from Oceans moves to |
| Option A: | Mountains |
| Option B: | Equator |
| Option C: | Plain areas |
| Option D: | Poles |
| Q97. | The wind intensity can be described by |
| Option A: | Reynolds number |
| Option B: | Mach number |
| Option C: | Beaufort number |
| Option D: | Froude number |
| Q98. | Instrument used to measure direct beam of solar irradiance is called ___________ |
| Option A: | Pyranometer |
| Option B: | Net radiometer |
| Option C: | Bourdon gauge |
| Option D: | Pyrheliometer |
| Q99. | Photovoltaic cell or solar cell converts |
| Option A: | Electromagnetic radiation directly into electricity |
| Option B: | Thermal energy into electricity |
| Option C: | Solar radiation into kinetic energy |
| Option D: | Solar radiation into thermal energy |
| Q100. | The zenith angle is the angle made by the sun’s rays with the ____ to a ______ surface. |
| Option A: | normal, horizontal |
| Option B: | tangent, horizontal |
| Option C: | normal, vertical |
| Option D: | tangent, vertical |
| Q101. | The voltage of a single solar cell is |
| Option A: | 0.2 v |
| Option B: | 1.0 v |
| Option C: | 0.5 v |
| Option D: | 2.0 v |
| Q102. | In which type of absorber plate is the overall flow direction is along with the absorber glass plate? |
| Option A: | Finned absorber |
| Option B: | - Overlapped glass absorber |
| Option C: | Non-porous absorber plate |
| Option D: | Porous absorber plate |
| Q103. | For what purpose are Gas heating collectors used? |
| Option A: | To trap solar radiance |
| Option B: | To act as a medium to help in conversion of sunlight to electrical energy |
| Option C: | Employed as solar air heaters |
| Option D: | They act as alternate panels in case of failure |
| Q104. | Approximately how much amount of temperature achieved in Paraboloidal Dish Collector? |
| Option A: | 1000℃ |
| Option B: | 2000℃ |
| Option C: | 3000℃ |
| Option D: | 2500℃ |
| Q105. | In which collector does air flow without any obstruction? |
| Option A: | Porous absorber plate |
| Option B: | Non-porous absorber plate |
| Option C: | Over lapped glass absorber |
| Option D: | Finned absorber |
| Q106. | In the paraboloid dish concept, the concentrator tracks the sun by rotating about |
| Option A: | One axes |
| Option B: | Two axes |
| Option C: | Five axes |
| Option D: | Three axes |
| Q107. | LST stands for |
| Option A: | Local standard time |
| Option B: | Local solar temperature |
| Option C: | Low surface temperature |
| Option D: | Land surface temperature |
| Q108. | _______ is a glazing which limits the radiation and convection heat losses. |
| Option A: | Selective surface |
| Option B: | Transparent cover |
| Option C: | Insulation |
| Option D: | Absorber plate |
| Q109. | What are provided to minimize heat loss? |
| Option A: | Absorber plate |
| Option B: | Surface plate |
| Option C: | Insulation |
| Option D: | Casing |
| Q110. | Maximum efficiency is obtained in |
| Option A: | Flat plate collector |
| Option B: | Line focusing collector |
| Option C: | Evacuated tube collector |
| Option D: | Paraboloid dish collector |